Architectural and engineering specifications

S6L

• The 2-way full range active loudspeaker system shall have one direct radiating neodymium magnet, 1.5-inch voice coil,   6-inch low frequency transducer and one 1-inch voice coil diameter, PEN  diaphragm neodymium magnet compression driver loaded onto a horn. Low frequency transducer shall be rated at a nominal impedance of 8 ohm (nominal impedance). High frequency transducer shall be rated at a nominal impedance of 8 ohm (nominal impedance).

• The usable bandwidth shall be 62Hz to 20kHz (-10dB).

• The system’s horizontal coverage shall be 70º, with vertical coverage of 70º.

• Maximum SPL continuous, measured at 1m under free field conditions shall be 115 dB. Maximum SPL peak, measured at 1m under free field conditions shall be 118 dB.

• Enclosure shall be bass-reflex type with one round duct.

• Enclosure cabinet construction shall consist of birch plywood. Dimensions shall be 230 mm (9.05 inch) wide, 400 mm (15.7 inch) high, 250 mm (9.8 inch) deep. Enclosure weight shall be 7.2 Kg.

• Enclosure cabinet finish shall consist of black or white matt high-resistance Polyurea® paint.

• The front enclosure shall be protected by a 1.5mm steel grille with acoustically transparent technical mesh. The grille shall be secured via screws to keep it in place when facing the cabinet downwards.

• The enclosure shall include M8 points for rigging and shall be used in conjunction with dedicated rigging accessories. Rigging accessories shall be affixed with special crop resistant screws.

• The amplifier shall be built inside the cabinet, consisting of two channels, with a total power of 200W/8Ohm for the low frequency channel and 50W/8Ohm for the high frequency channel. The amplifier shall be 3rd Generation Class D.

• The amplifier’s power supply shall be switching mode equipped with Power Factor Correction and accept a universal input voltage (85 to 265 VAC). The power supply shall protect itself in case of excessive voltage at the mains and rearm itself when the voltage is in the specified range. Mains supply shall be through PowerCon, and a PowerCon Link connector shall be available.

• The analogue audio input shall be electronically balanced, with a minimum input impedance of 50kOhm. Connectors shall be XLR type (one female for input and one male for linking). The nominal input shall be of +8dBu, and the input shall accept up to +20dBu without distortion.

• The system shall be equipped with an in-built digital signal processor (DSP), integrated to the power amplifier. The internal DSP architecture shall be 64-bit double precision, featuring IIR and FIR filters. Analogue-to-Digital converters shall have a signal-to-noise ratio better than 98dB, 24-bit word size and shall work at 48 kHz sampling rate.

• The DSP shall contain factory and the possibility of user-defined presets. The presets and the main DSP parameters such as gain, limiter threshold and delay shall be visible and selectable by the user at the cabinet’s rear panel via a built-in 3.5” TFT touchscreen. The available memory for the delay line shall allow up to 100m (300 milliseconds).

• The DSP board shall be equipped with an internal USB port for firmware update and maintenance purposes. The port shall be accessed by removing the amplifier from the cabinet. The connector used will be USB-C.

• The loudspeaker system shall be the Amate Audio® NITID S6L.

• The rigging accessories shall be the Amate Audio® HR-LU, HR-LU/GT, ACR-M8, SP-6N, SP-WHR, RB-S, RB-S/GT.

April 2024